QYE101R1B8 Industrial Tablet

Specifications

Ver.#: 1.0

2022.06

QIYANG TECHNOLOGY Co., Ltd

Copyright Reserved

Technical Support

If you have questions about the documents, you can contact us during office hours (Monday to Friday 8:30-12:00, 13:30-17:30)

Contact us at

Technical Email：supports@qiyangtech.com

Technical Support Telephone No.：0571-87858811-805

Official website：www.qiytech.com(Chinese）/www.qiyangtech.com(English）

Information updates

1. Information updates

Product-related manuals and datasheets are constantly being improved and updated; please ensure that they are up-to-date when you use them.

2.Update notice

Qiyang’s newest product information and news updates will be released through the WeChat official account, please pay attention!

3.How to get information

After purchase, please contact the relevant sales staff to obtain the SDK.

Version Record

| Version | Hardware Platform | Description | Date | Revisor |
| --- | --- | --- | --- | --- |
| 1.0 |  | First release | 2023-06 |  |

Contents

Contents

Ⅰ. Preface	1

1.1.Company Profile	1

1.2. Use Suggestion	1

Ⅱ. Appearance	2

Ⅲ. Parameters	3

Ⅳ. Structure &Size	5

Ⅴ. Pin Definition	6

Ⅵ.Installation Methods	9

Ⅶ. Shipping List	10

Ⅷ. Cautions	11

Notice: This manual introduces the hardware interface of the QYE101R1B8 Industrial Tablet.

Ⅰ. Preface

1.1.Company Profile

Zhejiang Qiyang Intelligent Technology Co., Ltd., established in 2007, which locates in Hangzhou, Zhejiang, PRC. It is a high-end technological enterprise that specializes in exploitation, fabrication, and selling embedded computer mainboards . With 10 years of experiences, Qiyang has established the completed service chain from the design concept to mass production successfully.

The R&D team is organized by 30 more technical engineers. Qiyang focus on providing functional embedded hardware, software tool and customization solutions. It has been applied to Industrial Control, Internet of Things, New Retail, Smart Medical, Electricity Device, Environmental Surveillance, Charging Pile etc.

With the growth of the business, Qiyang has set up an SMT factory in Zhuji, Zhejiang province, which is 5000㎡, with a 2xSMT production line. The SMT factory performs the ISO9001 Quality Management System strictly. Relying on the solid production ability, the SMT factory’s annual capacity is about a million sets, which totally guarantee the delivery date.

Qiyang has a thorough sales marketing network, professional sales ,and after-sales team to provide full technical support and service. The business has spread over 120 countries and areas, it helps the clients to introduce the products into the market efficiently and successfully.The combination and extension of research and development, production capacity, and market, that provide a solid foundation for Qiyang to provide specialized, globalized embedded hardware and software.

We offer:
l.Software/Hardware Mainboard

Based on the CPU solution from NXP,Rockchip,MTK,Renesas,TI,Atmel,Cirrus Logic etc, Qiyang provides the ARM development kit/system on module/industrial board and periphery products, paired tools and software for the user do further exploitation.

2.Customization Service

Fully taking the advantage of the technical accumulation on the ARM platform and Linux, Android, Ubuntu OS, Qiyang provides the efficient OEM/ODM service to the users.

Sincerely thanks for using Qiyang’s product, we will try our best to offer you the technical supports!

1.2. Use Suggestion

1)Please read this manual before using.

2)Please check the packing list, and accessories to ensure there is no missing;

3)Understand basic structure and composition of the products, including the hardware parameters, etc.

Ⅱ. Appearance

Front Side

Back Side

Ⅲ. Parameters

| Screen | Size | 10.1 inch, full view |
| --- | --- | --- |
|  | Resolution | 1280*800 |
|  | Brightness | 350 CD/M2 |
|  | Touch Panel | Resistive Touch Panel |
| Performance | SOC | NXP i.MX8M Plus processor |
|  | CPU | 4*ARM Cortex-A53@1.6Ghz + 1* Cortex-M7 @800Mhz |
|  | GPU | GC7000UL with OpenCL and Vulkan support; Support 16 GFLOPS (High Precision)OpenGL ES 3.0/3.1,Vulkan,Open CL 1.2FP,OpenVG1.1 |
|  | VPU | Support 1080p60, h.265/4,VP8,VP9 video decoding Support 1080p60, h.265/4 video encoding |
|  | NPU | Neural processor unit, 2.3 TOPS |
|  | Memory | 4GB LPDDR4, (8GB LPDDR4 optional) |
|  | Storage | 16GB eMMC (32GB eMMC/64GB eMMC) |
|  | PMIC | NXP PCA9450 PMU |
| Ports | Ethernet | 2-ch 10/100/1000Mbps Ethernet, RJ45 type |
|  |  | 2-ch 10/100Mbps Ethernet, RJ45 type |
|  | Communication | Debug UART (RS232), baud rate :115200bps, inbuilt |
|  |  | 1-ch RS232, isolated, baud rate: 9600bps~112500bps |
|  |  | 5-ch RS485, isolated, baud rate: 9600bps~112500bps |
|  |  | 3-ch CAN2.0, support CAN-FD, isolated , baud rate : 125kbps~1Mbps |
|  | USB | 1-ch USB-OTG（USB2.0）, Type-C |
|  |  | 1-ch USB-HOST（USB3.0）,Type-A |
|  | Storage | 1-ch TF card socket, to connect TF card, it supports 32GB/64GB TF memory card. |
|  |  | 1-ch M.2(M-KEY) port, to connect SSD externally, it supports 512GB/1TB SSD. |
|  | IO | 16-ch DI, photoelectric isolation digital input, every 4-ch DI shares a public terminal |
|  |  | 4-ch DO, relay output (5A in max.) |
|  | AD | 4-ch Analog input (12bit,1%), 0-5V (No isolation) |
|  |  | 4-ch ADC, to connect NTC temperature sensor (Accuracy ±1℃) |
|  | Expansion | 1-ch M.2 (B-KEY), to connect 4G/5G ;MICRO SIM card socket. |
|  | Others | I2C, available to be extended as DAC (AO port) |
|  | RTC | Hardware RTC |
|  | Other circuits | Hardware watchdog circuit |
|  | Indicator | Power indictor x1(Red), system running indictor x1 (Green) |
|  | Nominal Voltage | DC24V, DC9V~32V power input support |
| System | OS | Support Ubuntu 20.04 |
|  |  | Support Linux 6.1+Qt5.15 |
|  | Startup LOGO | Support to change LGO |
| Electricity | Nominal Voltage | DC24V, DC9V~32V power input support |
|  | Power Consumption | About 12W |
|  | Power Protection | With lighting and surge protection |
|  | Power Failure Protection | ＜5S |
| Application Environment | Operational Temperature | -20℃~70℃ |
|  | Storage Temperature | -40℃~85℃ |
|  | Humidity | 10%~90%RH（Non-condensing） |
|  | Heat Dissipation Method | Natural heat dissipation |
| Mechanical Specifications | Dimensions | 271mm x 190.6mm x 34.7mm |
|  | Weight | About 2.5KG |
|  | Material | Galvanized sheet + AL |
|  | Protection Level | Front plate complies with IP65, the rear cover complies with IP50 |
|  | Installation Method | Clip-on Installation, Wall-mounted Installation |

Ⅳ. Structure &Size

Device Dimensions:271mm x 190.6mm x 34.7mm。

Ⅴ. Pin Definition

Backside Connectors

Backside Connectors

PWR-IN: EDG series-Single Row 2P 5.08MM spacing with screw thread connector

PIN Definition:

| # | Pin Definition | Description |
| --- | --- | --- |
| 1 | PWR_IN | PWR-IN |
| 2 | GND_PWR_IN |  |

RS485&RS232:EDG Series-Double Row 18P 3.81MM spacing connector

PIN Definition:

|  | Pin Definition | # | # | Pin Definition |  |
| --- | --- | --- | --- | --- | --- |
|  | RS485_CH1_A | 1 | 2 | RS485_CH2_A |  |
|  | RS485_CH1_B | 3 | 4 | RS485_CH2_B |  |
|  | SHELL_EARTH | 5 | 6 | SHELL_EARTH |  |
|  | RS485_CH3_A | 7 | 8 | RS485_CH4_A |  |
|  | RS485_CH3_B | 9 | 10 | RS485_CH4_B |  |
|  | SHELL_EARTH | 11 | 12 | SHELL_EARTH |  |
|  | COM_RXD | 13 | 14 | RS485_CH6_A |  |
|  | COM_RXD | 15 | 16 | RS485_CH6_B |  |
|  | GND_ISO_C | 17 | 18 | SHELL_EARTH |  |

CAN:EDG Series-Double Row 8P 3.81MM spacing connector

PIN Definition:

|  | Pin Definition | # | # | Pin Definition |  |
| --- | --- | --- | --- | --- | --- |
|  | CAN1_H | 1 | 2 | SHELL_EARTH |  |
|  | CAN1_L | 3 | 4 | SHELL_EARTH |  |
|  | CAN2_H | 5 | 6 | CAN3_H |  |
|  | CAN2_L | 7 | 8 | CAN3_L |  |

DI:EDG Series-Double Row 20P 3.81MM spacing connector

PIN Definition:

|  | Pin Definition | # | # | Pin Definition |  |
| --- | --- | --- | --- | --- | --- |
|  | DIN1_1 | 1 | 2 | DIN3_1 |  |
|  | DIN1_2 | 3 | 4 | DIN3_2 |  |
|  | DIN1_3 | 5 | 6 | DIN3_3 |  |
|  | DIN1_4 | 7 | 8 | DIN3_4 |  |
|  | GND_DIN1_I/F | 9 | 10 | GND_DIN3_I/F |  |
|  | DIN2_1 | 11 | 12 | DIN4_1 |  |
|  | DIN2_2 | 13 | 14 | DIN4_2 |  |
|  | DIN2_3 | 15 | 16 | DIN4_3 |  |
|  | DIN2_4 | 17 | 18 | DIN4_4 |  |
|  | GND_DIN2_I/F | 19 | 20 | GND_DIN4_I/F |  |

D0:EDG Series-Double 8P 3.81MM spacing connector

PIN Definition

|  | Pin Definition | # | # | Pin Definition |  |
| --- | --- | --- | --- | --- | --- |
|  | RELAY2_OUT1 | 1 | 2 | RELAY1_OUT1 |  |
|  | RELAY2_OUT2 | 3 | 4 | RELAY1_OUT2 |  |
|  | RELAY3_OUT1 | 5 | 6 | RELAY4_OUT1 |  |
|  | RELAY3_OUT1 | 7 | 8 | RELAY4_OUT1 |  |

NTC&AI:EDG Series-Double 16P 3.81MM spacing connector

PIN Definition

|  | Pin Definition | # | # | Pin Definition |  |
| --- | --- | --- | --- | --- | --- |
|  | TEMP_NTC4 | 1 | 2 | GND |  |
|  | TEMP_NTC3 | 3 | 4 | GND |  |
|  | TEMP_NTC2 | 5 | 6 | GND |  |
|  | TEMP_NTC1 | 7 | 8 | GND |  |
|  | AD4 | 9 | 10 | GND |  |
|  | AD3 | 11 | 12 | GND |  |
|  | AD2 | 13 | 14 | GND |  |
|  | AD1 | 15 | 16 | GND |  |

Ⅵ.Installation Methods

Clip-on Installation:

Installation Illustration:

Wall-mounted Installation:

Mounting Hole: International VESA standard:75mm*75mm

Ⅶ. Shipping List

Shipping List

| # | Item | Description | Qty | Remark |
| --- | --- | --- | --- | --- |
| 1 | 10.1 Inch All-In-One Device | QYE101R1B8 | 1 |  |
| 2 | Certificate of Conformance |  | 1 |  |
| 3 | Accessories | Installation accessories, connection terminal | 1 |  |
| 4 | Packaging Carton |  | 1 |  |

Configuration

| # | Model No. | Description |
| --- | --- | --- |
| 1 | QYE101R1B8-1N00 | Basic version |
| 2 | QYE101R1B8-1N10 | 4G(CAT4) Module |
| 3 | QYE101R1B8-1N20 | 5G Module |

Ⅷ. Cautions

- Please use DC12V±5% or 24V±5% power adapter;

- Adding cover plate on the device is prohibited, it may cause touch failure;

- Applications should avoid writing large amounts of data frequently, to prevent the EMMC storage unit from reaching its lifespan limit and causing the system to fail.

- It is strictly forbidden to disassemble and repair the machine without permission.

Zhejiang Qiyang Intelligent Technology Co.,Ltd

Tel：0571-87858811 / 87858822

Fax：0571-89935912

Technical Support：0571-87858811转805

E-MAIL：supports@qiyangtech.com

Website：http://www.qiytech.com

ADD：3rd Floor, Building A, No.6 West Park 8th Road, West    Lake Science and Technology Park, West Lake District, Hangzhou, China

Post Code：310030